Bangladesh University of Engineering and Technology

Chemical Engineering Laboratory I

Course No: ChE 206

A Formal Report On

Calibration of Orifice Meter

Prepared for:

Mahmud Hayet Rudro

Lecturer

Department of Chemical Engineering, BUET

Prepared by:

Md. Tanvir Ahmmed

ID:2202026

Level-2, Term-2

Group:06

Partner’s ID:

2302026

2302027

2302028

2302029

2302030

Date of Performance: 22/08/2026

Date of Submission  : 05/08/2026

Statement of Academic Integrity

I hereby sincerely declare that I solely prepared this laboratory report titled “Calibration of Orifice Meter”. This report is an original work based on the experimental observations done by us. All the calculations, AutoCAD drawing, graphical analysis and interpretations are independently done by me. Any part of this report has not been copied, fabricated or submitted previously, for the purpose of any other academic assessment and all references used in this report have been cited properly.

I am fully aware of the academic integrity policies of the Department of Chemical Engineering and understand that any form of academic misconduct such as plagiarism, is subjected to disciplinary action as per departmental regulations.

I accept full responsibility for the authenticity, accuracy and integrity of the contents of this report.

Md. Tanvir Ahmmed

Student ID: 2202026

Date: 05/09/2026

Signature

Summary

Accurately determining fluid flow rates and evaluating discharge characteristics is essential in chemical engineering applications where orifice meters serve as reliable differential pressure devices. This study investigates the calibration and hydrodynamic performance of an orifice meter by running water through the system across 8 different flow rates to measure the scale readings and differential manometer heights. From this data, the volumetric flow rates, pressure drops, discharge coefficients Co and Reynolds numbers Re were analyzed through five characteristic curves: Q vs ΔH ,Q vs ΔH , Co vs ΔH , Co vs ΔH, and Co vs Re. Quantitatively, the water flow rates ranged from 3.76×10-4 m3/s to 6.72×10-4 m3/s .Meanwhile, the pressure head difference ,ΔH increased non linearly from 0.049 m H2O to 0.162 m H2O.This yields a strong parabolic fit with a R2 value of 0.9757  that successfully linearized upon square root transformation where the value of R2 is 0.98. Throughout the tests, Co remained stable between 0.722 and 0.783 as Reynolds numbers increased from 11414 to 20409 confirming a fully turbulent operating regime. However, the experimental o​ values deviated from the standard theoretical range 0.60–0.65. This happens because fast moving fluid creates friction along pipe walls and extra turbulence near pressure taps, while long term use has worn down the sharp edges of the orifice plate. These physical factors alter downstream pressure recovery and naturally push the measured discharge coefficients higher than ideal predictions. To improve future reliability and broaden the experimental scope, the study could have been expanded by testing multiple process fluids or comparing different orifice plate sizes to further validate the generalized calibration model across varied operational conditions.

Acknowledgement

This formal report is based on experiment 04 named ‘Calibration of Orifice meter’ from ChE-206 "Chemical Engineering Laboratory-I." The experiment was done on August 22, 2026, in the Fluid Mechanics Laboratory of Chemical Engineering Department, Bangladesh University of Engineering and Technology.

I want to give thanks to our teacher, Mahmud Hayet Rudro. His instructions and guidance helped us a lot to finish the experiment successfully. Also, his explanation on the theory made it easy for us to clear the concept. I am also grateful to him because he answered all our queries with patience.

During the lab, our lab assistant was really helpful. He described the full process to us step by step and also cautioned us so we don't make any silly mistakes. Without his kind assistance, it would be difficult for us to carry out the experiment properly and finally I am thankful to all of my group members for their cooperation and help.

Md. Tanvir Ahmmed

Student ID: 2202026

Section: A1

Group No: 06

Level-2, Term-2

Department of Chemical Engineering, BUET

Table of contents

Statement of Academic Integrity……………………………………………………………. ..i

Summary ……………………………………………………………………………………...ii

Acknowledgement…………………………………………………………………………....iii

Table of Contents……………………………………………………………………………..iv

List of Figures………………………………………………………………………………....v

List of Tables…………………………………………………………………………………..v

1. Introduction………………………………………………………………………………....1

2. Theory………………………………………………………………………………………1

2.1 Orifice Meter and It’s Working Principle……………………………………………….1

2.2 Vena Contracta…………………………………………………………………………..2

2.3 Classification of Orifice Plates…………………………………………………………..3

2.4 Continuity Equation……………………………………………………………………..4

2.5 Bernoulli's Equation……………………………………………………………………..4

2.6 Differential Manometer Relation………………………………………………………..5

2.7 Reynold’s Number………………………………………………………………………6

2.8 Coefficient of Orifice……………………………………………………………………6

2.9 Calibration of Orifice Meter…………………………………………………………….7

3.Apparatus and Materials…………………………………………………………………….9

3.1 Apparatus………………………………………………………………………………..9

3.2 Fluid……………………………………………………………………………………..9

3.3 Experimental Setup……………………………………………………………………..9

3.4 Experimental Procedure………………………………………………………………..10

4. Data collection……………………………………………………………………………..11

4.1 Observed Data………………………………………………………………………….11

4.2 Calculated Data………………………………………………………………………...12

4.3 Graphical Representation………………………………………………………………14

5.Result and Discussion……………………………………………………………………...17

6.Conclusion………………………………………………………………………………….19

List of References…………………………………………………………………………….21

Nomenclature………………………………………………………………………………...22

Appendices…………………………………………………………………………………...23

Appendix-A………………………………………………………………………………...23

Appendix-B………………………………………………………………………………...24

Appendix-C………………………………………………………………………………...27

List of Figures

Figure 1: Schematic diagram of an orifice meter……………………………………………..2

Figure 2: Different types of orifice plates based on geometry………………………………..3

Figure 3: Schematic diagram of orifice meter apparatus…………………………………….10

Figure 4: Plot of pressure head difference vs volumetric flow rate………………………….14

Figure 5: Plot of square root of pressure head difference vs volumetric flow rate…………..14

Figure 6: Plot of coefficient of discharge vs pressure head difference………………………15

Figure 7: Plot of coefficient of orifice vs square root of pressure head difference…………..15

Figure 8: Plot of coefficient of discharge vs Reynolds number……………………………...16

List of Tables

Table 1: Observed data for the volume of water, time interval and manometer reading…….11

Table 2: Data table for the calculated values of theoretical and experimental volumetric

flow rate, pressure head difference and square root of pressure head difference……………12

Table 3: Data table for the calculated values of average velocity, coefficient of orifice

and Reynold’s number………………………………………………………………………13

1.Introduction

Measurement of fluid flow rate is an important requirement in many engineering and industrial applications like chemical processing, power generation, water supply systems and petroleum industries. Among the various flow-measuring devices, the orifice meter is one of the simplest and most commonly used instruments due to its low cost, easy installation and reliable performance under turbulent flow conditions. An orifice meter consists of a thin plate with a circular opening inserted into a pipeline, which creates a pressure drop as the fluid passes through it. This pressure difference between the upstream and downstream sections of the orifice is directly related to the flow rate of the fluid.

The working principle of an orifice meter is based on the application of Bernoulli's equation and the continuity equation which relate velocity, pressure and flow area. The theoretical discharge obtained from these relations assumes ideal flow conditions without any energy losses. However, in real systems, losses occur due to friction, turbulence and flow separation near the orifice plate. To account for these deviations, a correction factor known as the coefficient of orifice (Co​) is introduced. Finding this coefficient through experimental calibration is therefore necessary for accurate flow measurement.

In this experiment, an orifice meter is calibrated by measuring the volumetric flow rate of water and the corresponding pressure head difference across the orifice. The experimental discharge is compared with the theoretical discharge to evaluate the coefficient of orifice and to study the relationship between pressure head difference and flow rate. The experiment also aims to verify the applicability of the theoretical flow equation under turbulent flow conditions and to assess the performance characteristics of the orifice meter as a practical flow-measuring device.

2.Theory

2.1 Orifice Meter and It’s Working Principle

An orifice meter is basically a simple device used in pipes to measure fluid flow. It has a thin metal plate with a small hole in the middle that creates a sudden narrowing, causing the fluid speed to increase and the pressure to drop right at that spot. Also this setup is fixed between pipe flanges securely to make sure no leakage happens during operation.

Figure 1: Schematic diagram of an orifice meter [1]

The fluid starts passing through the hole, the stream cannot change direction instantly. So, it keeps narrowing a bit more just after the plate to form a minimum cross-section area point called vena contracta where velocity is highest and pressure is lowest. Because of this sudden restriction, the fluid flow becomes very fast and turbulent right at that area before expanding back again.

To find the reading, we connect a differential pressure gauge or manometer across the upstream point and the vena contracta point to measure that pressure drop carefully. Since this pressure difference depends directly on how fast the fluid is moving through the pipe, we use that measured height difference to calculate the actual discharge rate with the help of standard flow equations.

2.2 Vena Contracta

When fluid goes from the big pipe into that small hole of the orifice plate, the liquid particles cannot change their direction fast. Because of this inertia problem, the fluid stream keeps getting narrower more even after passing the plate until it makes a smallest area spot, and this exact place is called the vena contracta. Liquid cannot adjust instantly so it takes some distance after the plate to reach this minimum width area.

At this vena contracta spot, the fluid speed goes very high to maximum level and the static pressure drops down to lowest point. Also in this area, the fluid flow becomes totally rough and chaotic with many small eddies before it slowly spreads back to fill the normal pipe size. Due to this sudden change, energy loss also happens there before the flow normalizes again in downstream.

2.3 Classification of Orifice Plates

Different types of orifice plates are used in industrial pipelines depending on the nature of the fluid and the specific operating conditions. The most common and standard type is the concentric orifice plate, where the circular hole is placed exactly at the middle of the plate. This design is used mostly for clean water or clear fluids because it provides a proper, smooth flow contraction and a clear pressure drop that is easy to calculate.

Figure 2: Different types of orifice plates based on geometry [2]

When the fluid contains solid dirt, mud, or gas bubbles, a standard concentric plate can cause problems like sediment blocking at the bottom or gas getting trapped. For those types of situations, we use an eccentric orifice plate, which has an off-center hole so solid particles and impurities can easily pass through the bottom without getting stuck. Another similar design is the segmental orifice plate, which has a D shape opening at one side and is also very helpful when the liquid contains heavy solid materials or air bubbles.

Sometimes conical orifice plates are also used because they have a special conical inlet profile to give a smoother flow entry with less energy loss. Besides, when a very high flow rate creates too much vibration and loud noise in the pipeline, we can use a multi-hole orifice plate, which has several small holes distributed across the plate instead of one big center hole. All these different plate geometries help us measure flow correctly and safely without getting the pipe blocked, damaged, or making too much vibration.

2.4 Continuity Equation

For any steady and incompressible fluid flow, the continuity principle basically tells us that mass flow rate always stays constant from one point to another. Mathematically, it is written as:

A1v1=A2v2

where,

A=Cross sectional area of pipe

v=Average velocity of fluid

Inside an orifice meter setup, when the liquid passes through that small restriction hole, the flow space suddenly gets smaller. Based on the continuity relation, when area goes down, fluid speed has to go up at that exact spot. This sudden jump in velocity causes the static pressure to drop down heavily. So, we can say that the continuity equation basically shows why the speed increases at the narrow throat, which is the main reason behind getting that pressure difference for flow measurement.

2.5 Bernoulli's Equation

Bernoulli's equation for steady, frictionless and incompressible fluid flow along a streamline combines pressure, velocity and elevation energy.

It is expressed as:

P1ρg+v122g+z1=P2ρg+v222g+z2

Since the orifice meter is set up horizontally in the pipeline, the height terms (z1 and z2) on both side are equal so they just cancel out each other completely. Because of that, equation becomes simple like:

P1-P2ρg=v22-v122g

Here,P1 and P2 means static pressures at upstream and downstream section. As pipe is horizontal and height does not change at all, pressure difference is totally balanced by velocity head change. This relation gives main theoretical basis for measuring fluid flow using pressure devices like the orifice meter.

2.6 Differential Manometer Relation

To find out the pressure difference across the orifice plate, a U-tube differential manometer is used which contains carbon tetrachloride (CCl4) as the manometric liquid. From the manometer deflection, the equivalent head difference in terms of water is calculated using this formula:

ΔH=R (ρccl4ρw-ρw)

where,

R = Manometer reading difference

ρccl4 = Density of manometer fluid

ρw = Density of water

When liquid flows through the orifice, the restricted area raises the fluid speed and drops the static pressure. We connect the differential manometer across the upstream and downstream taps to capture this pressure drop. Then, the raw reading from the manometer gets converted into equivalent water head, which is directly used to find out the final discharge rate through the meter.

2.7 Reynold’s Number

The Reynolds Number (Re) is a dimensionless parameter in fluid mechanics representing the ratio of inertial forces to viscous forces within a moving fluid.

Re=Dvρμ

where,

ρ = Density of fluid

v = Average velocity

D = Diameter of pipe

μ = Dynamic viscosity of fluid

In an orifice meter experiment, the Reynolds number plays a very important role. The discharge coefficient (Co) of the orifice meter depends directly on the flow conditions and the Reynolds number. When the flow rate changes, the Reynolds number also changes, which affects the value of Co. Therefore, calculating the Reynolds number is essential to know the flow regime and to ensure accurate calculations during the experiment.

2.8 Coefficient of Orifice

In an ideal setup, the theoretical discharge through an orifice meter assumes no energy losses. However, in practical flow, actual discharge always stays lower than the theoretical value due to fluid friction losses, turbulence and the sudden contraction of the fluid at the vena contracta. To cover up this difference and make our calculations match with practical data, we use a term called the discharge coefficient, which is generally written as Co.

Mathematically, it is defined as the ratio of actual discharge to the theoretical discharge:

Cd=QexperimentalQtheoretical

In terms of lab parameters, the experimental discharge is calculated using this relation:

Qexperimental=CoAo2gΔH1-DoD4

Hence, rearranging the formula gives the final expression for Co:

Co=QexperimentalAo2gΔH1-DoD4  = VtAo(2gΔH1-DoD4) = VAot2gΔH1-DoD4

where,

Qexperimental = Real volume of fluid passing per second measured in the lab setup

Qtheoretical = Ideal flow rate calculated from the Bernoulli and continuity relations

V = Volume of water collected

t = Required time

The value of Co is always less than one, usually staying around 0.6 to 0.65 for normal orifice meters. It depends on several things like the Reynolds number of the flow, the size of the pipe and the ratio of the orifice hole diameter to the main pipe diameter. Moreover, if continuous fluid flow makes the sharp edges of the orifice plate smooth over time, the flow contraction decreases and the value of Co can increase. Finding the correct Co value is very important because it helps to bridge the gap between ideal theory and practical experimental results.

2.9 Calibration of Orifice Meter

An orifice meter requires physical calibration because baseline equations rely on frictionless, ideal flow assumptions. Real operations face boundary layer viscosity, vortex shedding, and vena contracta pressure shifts that alter the expected discharge values. This procedure establishes the precise empirical link between the differential pressure head and actual volumetric flow rate.

In this experiment, calibration is carried out across 8 different flow rates by determining actual discharge from recorded volumetric water collections over a known time interval and noting the corresponding pressure head differences ΔH via a manometer. Using this experimental data, five characteristic plots are prepared: flow rate versus pressure head difference Q vs ΔH, flow rate versus square root of pressure head difference Q vs ΔH, discharge coefficient versus pressure head difference Co vs ΔH , discharge coefficient versus square root of pressure head difference Co vs ΔH and discharge coefficient versus Reynolds number Co vs Re.

3. Apparatus and Materials

3.1 Apparatus

1. Orifice Meter

2. Differential U-tube manometer

3. Flowmeter

4. Centrifugal pump

5. Reservoir (water tank)

6. Connecting pipes

7. Pipe fittings (elbows, joints, connectors)

8. Manometer stand

9. Scale attached with manometer

10. Stopwatch

11. Thermometer

3.2 Fluids

12. Water

13. Manometer Fluid (CCl4)

3. Experimental Setup

The experimental arrangement is built around a closed loop piping network connected to a cylindrical water reservoir. A centrifugal pump is fitted at the suction line with an entrance valve to draw water from the tank. The discharge line from the pump runs through a network of pipes containing a bypass valve, a main valve, a pressure control valve and a flowmeter before reaching the test section. The test section has an orifice plate that creates a controlled flow restriction. Two pressure tapping lines are taken from the upstream and downstream sides of this orifice plate and connected to a vertical differential U-tube manometer. A graduated scale is mounted beside the manometer column to accurately measure the liquid deflection and determine the pressure head difference during the run.

Figure 3: Schematic diagram of orifice meter apparatus

3.4 Experimental Procedure:

The experiment starts with opening the supply valve to fill the storage reservoir with water, providing an uninterrupted fluid supply to the centrifugal pump. The entrance valve and the bypass valve were then adjusted to maintain a steady, uniform flow rate while setting the upper flow limit and avoiding any tank overflow. Before noting down any readings, the differential U-tube manometer was checked to make sure both fluid arms were balanced at the same initial zero level. After making sure the pressure tap links were secure, the control valve was tuned to set the initial flow rate, and enough time was given for the flow to become stable before recording data. To find the experimental discharge, a fixed volume of water equal to half a cycle of the flowmeter 5L was collected and the exact time interval was tracked using a stopwatch. Under these identical operating conditions, the liquid column heights in the left and right limbs of the manometer were noted to calculate the pressure head difference across the orifice plate. This entire measurement cycle was repeated for seven additional runs at gradually higher flow rates. Finally, both the room temperature and the water temperature were recorded using a thermometer to determine the required physical properties like density and viscosity for the water and carbon tetrachloride.

4. Data Collection

4.1 Observed Data

Room temperature = 29.5°C

Temperature of water = 30°C

Table 1: Observed data for the volume of water, time interval and manometer reading

| No. of Observation | Volume, V(L) | Time, t(s) | Manometer Reading of CCl4 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Left(cm) | Right(cm) | Difference(cm) |
| 1 | 5.0 | 13.3 | 33.0 | 24.7 | 8.3 |
| 2 |  | 11.6 | 34.5 | 23.5 | 11.0 |
| 3 |  | 10.2 | 36.0 | 22.0 | 14.0 |
| 4 |  | 9.09 | 37.5 | 20.3 | 17.2 |
| 5 |  | 8.03 | 38.7 | 19.0 | 19.7 |
| 6 |  | 7.94 | 39.7 | 17.7 | 22.0 |
| 7 |  | 7.69 | 40.6 | 16.5 | 24.1 |
| 8 |  | 7.44 | 42.5 | 15.5 | 27.0 |

4.2 Calculated Data

Density of water at 30°C, ρw = 995.82 kg/m3 [3]

Density of CCl4 at 29.5°C, ρccl4 = 1594 kg/m3 [3]

Viscosity of water at 30°C, µw = 7.97×10-4 Pa.s [3]

Table 2: Data table for the calculated values of theoretical and experimental volumetric flow rate, pressure head difference and square root of pressure head difference.

| No.of Observation | Time, t(s) | Theoretical Volumetric Flow Rate, QTheo×104 (m3/s) | Experimental Volumetric Flow Rate, QExp×104 (m3/s) | Manometer Reading Difference, R (cm) | Pressure Head Difference, ∆H (m H2O) ×102 | ΔH (m H2O) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 13.3 | 5.16 | 3.76 | 8.30 | 4.99 | 0.223 |
| 2 | 11.6 | 5.94 | 4.31 | 11.0 | 6.61 | 0.257 |
| 3 | 10.2 | 6.70 | 4.90 | 14.0 | 8.41 | 0.290 |
| 4 | 9.09 | 7.43 | 5.50 | 17.2 | 10.3 | 0.321 |
| 5 | 8.03 | 7.95 | 6.23 | 19.7 | 11.8 | 0.344 |
| 6 | 7.94 | 8.40 | 6.30 | 22.0 | 13.2 | 0.363 |
| 7 | 7.69 | 8.79 | 6.50 | 24.1 | 14.5 | 0.381 |
| 8 | 7.44 | 9.30 | 6.72 | 27.0 | 16.2 | 0.403 |

Table 3: Data table for the calculated values of average velocity, coefficient of orifice and Reynold’s number

| No. of Observation | Average Velocity, v(m/s) | Coefficient of Orifice, Co | Reynold’s Number, Re |
| --- | --- | --- | --- |
| 1 | 0.174 | 0.729 | 11414 |
| 2 | 0.200 | 0.726 | 13090 |
| 3 | 0.227 | 0.732 | 14887 |
| 4 | 0.255 | 0.741 | 16705 |
| 5 | 0.288 | 0.783 | 18910 |
| 6 | 0.292 | 0.750 | 19124 |
| 7 | 0.301 | 0.740 | 19746 |
| 8 | 0.311 | 0.722 | 20409 |

4.3 Graphical Representation

Figure 4: Plot of pressure head difference vs volumetric flow rate

Figure 5: Plot of square root of pressure head difference vs volumetric flow rate

Figure 6: Plot of coefficient of discharge vs pressure head difference

Figure 7: Plot of coefficient of orifice vs square root of pressure head difference

Figure 8: Plot of Coefficient of discharge vs Reynolds number

5. Result and Discussion

Table 4: Data for volumetric flow rate, pressure head difference, coefficient of orifice and Reynold’s number

| No. of Observation | Experimental Volumetric Flow Rate, QExp×104 (m3/s) | Pressure Head Difference, ∆H (m H2O) | Square Root of Pressure Head Difference, ΔH (√m H2O) | Coefficient of Orifice, Co | Reynold’s Number, Re |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.76 | 4.99 | 0.0499 | 0.729 | 11414 |
| 2 | 4.31 | 6.61 | 0.0661 | 0.726 | 13090 |
| 3 | 4.90 | 8.41 | 0.0841 | 0.732 | 14887 |
| 4 | 5.50 | 10.3 | 0.103 | 0.741 | 16705 |
| 5 | 6.23 | 11.8 | 0.118 | 0.783 | 18910 |
| 6 | 6.30 | 13.2 | 0.132 | 0.750 | 19124 |
| 7 | 6.50 | 14.5 | 0.145 | 0.740 | 19746 |
| 8 | 6.72 | 16.2 | 0.162 | 0.722 | 20409 |

In this experiment, the working principle of an orifice meter was studied and its calibration was successfully performed. The whole calibration method was based on finding the relation between the volumetric flow rate and the pressure drop across the orifice plate. Because of this, the discharge coefficient (Co) could be easily determined at different flow rates to check the meter's performance under turbulent conditions.

The first graph illustrates the relationship between the pressure head difference, ΔH and the volumetric flow rate Qexp​, showing a clear parabolic trend where the pressure drop increases non linearly. The flow rate goes up from 3.76×10−4 to 6.72×10−4 m3/s with ΔH ranging from 0.0499 to 0.162 m H2​O and an R2 value of 0.9757. This parabolic shape happens because the flow rate through an orifice meter is proportional to the square root of the pressure drop, meaning ΔH increases with the square of the flow rate. To make this easier to analyze, the second graph illustrates the linear calibration plot of the square root of pressure head difference, ΔH​ versus Q, transforming that curved path into a straight line with an R2 value of 0.98 where ΔH​ values rise from 0.223 to 0.403. This linear trend proves that our experimental data follows the standard theoretical equation for an orifice meter very well, meaning the measurements are reliable and consistent across the whole testing range.

The third graph illustrates the plot of coefficient of discharge Co​ versus the pressure head difference ΔH, showing a steady horizontal trend where the discharge coefficient remains nearly constant between 0.722 and 0.783 as ΔH ranges from 0.0499 to 0.162 m H2​O. Following this, the fourth graph illustrates the plot of the coefficient of orifice versus the square root of pressure head difference ΔH​, where the data points maintain that exact same stable, flat pattern as varies from 0.223 to 0.403 m  H2O. Looking at the general trend, both graphs indicate that the discharge coefficient does not change with the pressure drop or flow velocity, which confirms the consistent performance of the meter under fully turbulent conditions across all trials. However, our experimental Co​ values are somewhat higher than the standard theoretical range of 0.60–0.65. This happens mainly because long term use has caused the sharp inlet edges of the orifice plate to wear down and become smooth. This minor edge wear reduces flow contraction and alters the pressure recovery behavior. It naturally pushes our measured discharge coefficients higher while still showing reliable and steady trends throughout the experiment.

The fifth graph illustrates the plot of the coefficient of discharge, Co​ versus the Reynolds number Re​. It displays a steady horizontal trend where the discharge coefficient remains constant as the Reynolds number increases from 11414 to 20409. This flat pattern means that Co​ becomes independent of Re​ at higher flow velocities, confirming that the system operates in a fully turbulent regime with stable and consistent meter performance.

According to the fundamental fluid mechanics theory, the volumetric flow rate, Q through an orifice meter must follow a strict square root relationship with the pressure head difference ΔH, meaning the flow rate is proportional to ΔH. Furthermore, under fully turbulent operating conditions, the discharge coefficient Co should remain constant and become entirely independent of the pressure head, square root of pressure head, and Reynolds number, Re.

But if the theoretical expectations are compared with the actual experimental data from all five graphs, consistent trends and minor offsets can be observed.

- Theoretical Behavior: Ideal equations assume frictionless flow, perfect contraction, and a standard discharge coefficient 0.60–0.65, yielding an exact predictable curve and flat Co trends.

- Experimental Data: The measured pressure head difference ΔH starts at 0.0499 m H2O for a flow rate of 3.76×10-4 m3/s and scales up non linearly to 0.162 m H2O at 6.72×10-4 m3/s displaying a strong parabolic fit R2 = 0.9757. Meanwhile, the experimental Co values across the Co vs ΔH, Co vs ΔH  and Co vs Re graphs consistently hover between 0.722 and 0.783 while maintaining a stable, flat horizontal trend as Reynolds numbers increase from 11414 to 20409.

It can be seen that while the experimental data matches the expected flat trend and confirms fully turbulent, stable meter performance. But the measured Co values are higher than strict textbook models and the pressure drops run slightly higher than ideal models. This discrepancy happens because the theoretical equations assume a completely sharp edged orifice plate and frictionless boundaries. In the actual laboratory setup, fast moving fluid creates friction along the pipe walls and extra turbulence near the pressure taps. Also, long term use has worn down the sharp edges of the orifice plate. Because the orifice plate's inlet edges have smoothed out over time, flow contraction is reduced, altering the pressure recovery and naturally pushing the experimental discharge coefficients higher than pure theoretical values. Additionally, minor manual or reading fluctuations during data collection can contribute slight variations.

Despite these real-world deviations, transforming the data into a plot of ΔH versus Q successfully yields a straight line and the constant horizontal trends across the discharge coefficient graphs prove that the system obeys the governing laws of fluid mechanics in turbulent regimes. The high repeatability and consistent patterns across all five graphs confirm that the orifice meter is reliable, highly repeatable, and well-suited for accurate industrial flow measurement once properly calibrated.

6.Conclusions

- The orifice meter was successfully calibrated across 8 different flow rates, establishing a reliable empirical relationship between the pressure head difference and the actual volumetric flow rate.

- The volumetric flow rate showed a nonlinear parabolic trend with pressure head difference which successfully linearized upon square root transformation, closely matching theoretical expectations.

- The discharge coefficients Co remained stable between 0.72 and 0.783 as Reynolds numbers increased from 11414 to 20409, confirming a fully turbulent operating regime.

- The experimental Co values deviated higher than the standard theoretical range of 0.60–0.65 due to real world pipe wall friction, localized turbulence near the pressure taps and physical edge wear on the orifice plate from long term use.

- The characteristic plots of Co versus ΔH and Co versus ΔH verified that the discharge coefficient maintains consistency and becomes independent of pressure drop magnitude within the turbulent zone.

- Expanding future testing to include multiple process fluids or varying orifice plate sizes would further strengthen and validate the generalized calibration model across broader operational windows.

- Once calibrated, the orifice meter can be used repeatedly to determine unknown flow rates directly from measured pressure drops without volumetric testing, saving time and improving industrial measurement consistency.

List of References

[1] "Orifice Meter Schematic Diagram," eclass.upatras.gr, Accessed: Aug. 30, 2026. [Online]. Available: https://share.google/foGdULltyoZADGZZI

[2] "Orifice Meter Lab Data & Manual," eclass.upatras.gr, Accessed: Aug. 30, 2026. [Online]. Available: https://share.google/BAqDvZS63JDpUJ5ft

[3] D. W. Green and R. H. Perry, Perry's Chemical Engineers' Handbook, 8th ed., McGraw-Hill, Accessed: Sep. 3,2026. Available: https://archive.org/details/perryschemicalen0000unse

[4] Finnemore, E. J., and Franzini, J. B., 2002. Fluid Mechanics with Engineering Applications, 10th edition, McGraw-Hill LLC. [Online]. Available: https://archive.org/details/fluidmechanicswi0000finn

[5]. Department of Chemical Engineering, Laboratory Manual for Experiment No. 4: Calibration of Orifice Meter.

Nomenclature

| Symbol | Description | Unit (SI) |
| --- | --- | --- |
| Q | Volumetric flow rate | m³/s |
| QExp | Experimental volumetric flow rate | m³/s |
| QTheo | Theoretical volumetric flow rate | m³/s |
| ΔH | Pressure head difference across orifice | m |
| R | Manometer reading difference | m |
| Ao | Area of orifice | m² |
| A | Cross-sectional area of pipe | m² |
| Do | Diameter of orifice | m |
| D | Diameter of pipe | m |
| v | Average velocity of fluid in pipe | m/s |
| Co | Coefficient of orifice | (dimensionless) |
| Re | Reynolds number | (dimensionless) |
| ρ | Density of fluid | kg/m³ |
| ρₘ | Density of manometer fluid (CCl₄) | kg/m³ |
| μ | Dynamic viscosity of fluid | N·s/m² |
| g | Acceleration due to gravity | m/s² |
| V | Volume of collected fluid | m³ |
| t | Time interval for collection | s |
| P₁ | Pressure at upstream section | N/m² |
| P₂ | Pressure at downstream section | N/m² |
| z₁ | Elevation head at upstream section | m |
| z₂ | Elevation head at downstream section | m |

Appendices

Appendix A

Sample Calculation for Observation No.1

Density of water at 30°C, ρw = 995.82 kg/m3 [3]

Viscosity of water at 30°C, µw = 7.97×10-4 Pa.s [3]

Density of CCl4 at 29.5°C, ρccl4 = 1594 kg/m3 [3]

Orifice Diameter, Do =1 inch= 0.0254 m

Diameter of pipe, D = 2.067 inch =0.0525 m; Schedule No.40 [4]

Area of Orifice, Ao = π×Do24=3.1416×0.025424=5.067×10-4 m2

Inner area of Pipe, A =π×D24=3.1416×0.052524= 2.16× 10-3 m2

Volume of water after a half cycle of flow meter, V = 5.0 ×10-3 m3

Time interval for a half cycle of flow meter, t = 13.3 s

Experimental Volumetric Flow rate of water, QExp = Vt =5.0×10-313.3= 3.76 ×10-4  m3/s

Average Velocity of water, v = QExpA =3.76×10-42.16×10-3= 0.174 m/s

Pressure Head difference, ∆H = R ×ρCCl4ρw -1

= 8.30 ×10-2 ×1594995.82-1 m H2O

= 0.05 m H2O

Square Root of Pressure Head Difference, ΔH = 0.224 m H2O12

Theoretical Volumetric Flow rate of water, QTheo=Ao2gΔH1-DoD4

= 5.067 ×10-4 × 2×9.81×0.051-0.02540.05254  m3/s

= 5.16 ×10-4 m3/s

Coefficient of Orifice, Co = QExpQTheo = 3.76×10-45.16 ×10-4 = 0.729

Reynolds Number, Re = Dvρw μw = 0.0525 × 0.174 ×995.82 7.97×10-4 = 11414

Appendix B

Let section (1) be the upstream pipe section and section (2) be the downstream pipe section of orifice meter.

From the Continuity Equation,

A1v1=Aov2

Where,

Inner area of pipe A1=πD24 
Area of orifice, Ao=πDo24

Thus,

v1=AoA1v2
v1=DoD2v2

Applying Bernoulli’s equation between section (1) and section (2),

P1ρg+v122g=P2ρg+v222g
Rearranging,

P1-P2ρg=v22-v122g
Let,

ΔH=P1-P2ρg
Then,

ΔH=v22-v122g
From continuity,

v1=DoD2v2
v12=DoD4v22
Substituting,

ΔH=v22-DoD4v222g
ΔH=v221DoD42g
Velocity through the Orifice,

v22=2gΔH1-DoD4
v2=2gΔH1-DoD4

The theoretical volumetric flow rate,

QTheoretical=Aov2
QTheoretical=Ao2gΔH1-DoD4
And,

Qactual=QExperimental=Vt
where,
V = Volume of fluid
t = Time interval

The theoretical discharge equation assumes ideal conditions with no energy losses. However, in real flow, losses occur due to viscosity, turbulence, flow separation, and vena contracta formation. Therefore, the actual discharge is always less than the theoretical discharge.

To account for these effects, the coefficient of orifice Cois introduced,

QExperimental=CoAo2gΔH1-DoD4

Hence,

Co=QExperimentalAo2gΔH1-DoD4=V/tAo2gΔH1-DoD4
For an ideal fluid, Co=1, but in practice its value is always less than unity.

Appendix C

Industrial Advantages and Disadvantages of Orifice Meters

Advantages

- Simple Design: Orifice meter design is very simple because it only uses a standard metal plate with a hole and a manometer to show pressure drop.

- No External Power: It does not need electricity or batteries to work since fluid pressure does behind the measurement, so it is safe for normal plant use.

- Low Cost: Compared to big digital flow meters, buying and setting up an orifice plate system is cheap and budget-friendly.

- Predictable Math: The relationship between square root of pressure head and flow rate follows a clear straight-line trend for easy calculation.

- No Moving Parts: Because there are no rotating parts inside the pipe, it does not get jammed easily and lasts for a long time.

Disadvantages

- Pressure Loss: After the fluid passes through the hole, it never gets back all its pressure, which wastes some pumping power permanently.

- Fluid Dependency: If we change the liquid from water to another chemical, the calibration changes and we have to recalculate everything.

- Edge Damage: If dirty water or abrasive particles flow through for many days, the sharp inner edge of the hole gets worn out, causing reading errors.

- Flow Range Limit: It only works well within a small flow range because pressure drop changes with the square of the velocity.

- Straight Pipe Needed: We must install enough straight pipe before and after the meter to remove all pipe bends and flow swirls.

| Assessment Component | Weight (%) | Obtained Marks |
| --- | --- | --- |
| Summary | 10% |  |
| Experimental Set up | 15% |  |
| Result and Discussion and Conclusion | 25% |  |
|  | Subtotal (50%) |  |
| Introduction and Theory | 15% |  |
| Reference and Nomenclature | 05% |  |
| Quality of Tables | 10% |  |
| Quality of Figures | 10% |  |
| Overall Presentation | 10% |  |
| Total | 100% |  |

Marking scheme:

A long report on ‘Calibration of Orifice Meter’

Name: Md. Tanvir Ahmmed

ID:2202026