BANGLADESH UNIVERSITY OF ENGINNERING AND TECHNOLOGY

ChE 206: Chemical Engineering Laboratory I

A Long Report on

Experiment No 04:  CALIBRATION OF ORIFICE METER

Submitted To:

Mahmud Hayet Rudro

Lecturer, Department of Chemical Engineering

Bangladesh University of Engineering and Technology, Dhaka-1000

Submitted By:

Tajrian Elmi Rashid

Student Id: 2302034

Section: A2

Level 2, Term 2

Lab Group: 01

Date of Performance:19/08/2026

Date of Submission: 09/09/2026

Lab Partners:

Abu Hossain Mohammed Abed Shihab,ID:2302031

Md Jamil Hasan, ID:2302032

Mahabubur Rahman Smoron, ID:2302033

Md Arif Faisal Abir, ID:2302035

# Statement of Academic Integrity

In signing this statement, I hereby certify that the work done on this long report titled “Calibration of Orifice Meter” is solely my own. I also declare that I have not copied from the work of any other person while completing this experiment and while preparing this report. Information collected from textbooks or other sources has been appropriately referenced and cited. I understand that if I fail to honor this agreement and violate academic honesty, I will be disqualified and subject to possible disciplinary action.

Name: Tajrian Elmi Rashid
Student ID: 2302034
Date: 9 September, 2026

_________________
         Signature

# Summary

The experiment was carried out to calibrate an orifice meter and to verify the theoretical relationships governing its operation. The objectives were to determine the coefficient of orifice experimentally, to establish the relationship between pressure head difference and volumetric flow rate, and to verify that the flow rate varies with the square root of the head under fully turbulent conditions. Water was circulated by a centrifugal pump through a 0.0525 m horizontal pipe fitted with a 0.0254 m orifice plate, and eight steady flow rates were set using the pressure control valve. The parameters obtained from the experiment were the volume of water collected, the time interval of collection, and the manometer reading across the plate, along with the water temperature of 29 °C, from which the density and viscosity of water were fixed. From these, the volumetric flow rate, pressure head difference, Reynolds number and coefficient of orifice were calculated for each run. The flow rate ranged from 3.81 × 10⁻⁴ to 6.99 × 10⁻⁴ m³/s, with the corresponding head rising from 4.19 × 10⁻² to 13.61 × 10⁻² m H₂O, and Reynolds numbers between 11,300 and 20,735 confirmed that the flow remained fully turbulent throughout. The head was found to increase parabolically with discharge and gave a straight line when plotted against √ΔH, verifying the expected relationship. The coefficient of orifice remained nearly constant across the range, varying between 0.797 and 0.851 with an average value of 0.819. This average is considerably higher than the 0.60–0.65 usually obtained from a sharp-edged plate, the most likely cause being wear and rounding of the upstream edge of the orifice after prolonged use, which weakens the contraction of the jet. Further error arose from the manometer, which could be read only to ±0.5 cm and whose fluid was continuously disturbed by vibration from the centrifugal pump. It is recommended that the orifice plate be inspected and replaced if edge wear is confirmed, that the manometer be isolated from pump vibration by mounting it on a damped support, and timing be automated or repeated at each setting to reduce random error. Lower flow rates should also be tested, so that the point at which the coefficient becomes constant can be located more accurately.

# Acknowledgement

The experiment ‘Calibration of Orifice Meter’ was conducted for the coursework of ChE206: Chemical Engineering Laboratory I, in Fluid Mechanics Laboratory. I heartily express my gratitude to the Department of Chemical Engineering, BUET for providing me with proper academic support and necessary laboratory infrastructural facilities. I would like to thank our teacher Mahmud Hayet Rudro for his kind guidance and instructions to understand the related concepts and experimental procedures. I also thank the lab instructor for helping with the experimental setup and equipment. Finally, my appreciation goes to my lab partners for their cooperation and teamwork, ensuring the successful execution of the experiment.

Tajrian Elmi Rashid
Student ID: 2302034
Department of Chemical Engineering
Bangladesh University of Engineering and Technology

# Table of Contents

Statement of Academic Integrity	i

Summary	ii

Acknowledgement	iii

Table of Contents	iv

List of Figures	v

List of Tables	v

1. Introduction	1

2. Theory	2

2.1 Orifice Meter and its Principle of Operation	2

2.2 Vena Contracta	2

2.3  Bernoulli’s Equation	3

2.4 Continuity Equation	3

2.5 Differential Manometer Relation	4

2.6 Derivation of the Governing Equation	5

2.7 Coefficient of Orifice	6

2.8 Types of Orifice Plates	6

2.9 Reynold’s Number	7

3. Materials and Methods	8

3.1 Apparatus	8

3.2 Experimental Setup	8

3.3 Experimental Procedure	9

4. Results and Discussions	10

4.1 Calibration Data of the Orifice Meter	10

4.2 Calibration Curves of the Orifice Meter	11

11

5. Conclusion	16

References	17

List of Symbols	18

Appendices	19

Appendix A	19

Observed Data	19

Appendix B	20

Calculated Data	20

Appendix C	21

Sample Calculation	21

# List of Figures

Figure 1: Diagram of an Orifice Meter	2

Figure 2: Different types of orifice plates based on geometry	8

Figure 3: Plot of Pressure head difference vs Volumetric flowrate	11

Figure 4: Plot of Square root of pressure head difference vs Volumetric flowrate	12

Figure 5: Plot of Coefficient of orifice meter vs Pressure head difference	13

Figure 6: Plot of Coefficient of orifice meter vs Square root of pressure head difference	14

Figure 7: Plot of Coefficient of orifice meter vs Reynold's number	15

# List of Tables

Table 1: Data for Volumetric Flow Rate, Pressure Head Difference, Reynold's Number and Coefficient of Orifice	10

Table 2: Observed Data for Volume of Water, Time Interval and Manometer Reading	17

Table 3: Table for Calculated Data of Volumetric Flow Rate, Velocity and Reynold's Number	18

Table 4: Table for Calculated Data of Pressure Head Difference, Root of Pressure Head Difference and Orifice Coefficient	19

# 1. Introduction

Measuring the rate at which a fluid flows through a pipe is a routine requirement in process engineering, since flow rates govern reaction stoichiometry, heat duties, pump selection and the accounting of raw materials. Direct measurement in a closed pipeline is hardly convenient, so the usual approach is indirect; an obstruction is placed in the line and the pressure difference it produces is measured instead. The orifice meter is the simplest device of this type, consisting of a thin plate with a machined hole clamped between pipe flanges, with pressure tappings on either side. It is inexpensive, has no moving parts and is easily replaced, though it causes a substantial permanent pressure loss and becomes insensitive at low flow rates.

Its operation follows from Bernoulli's theorem and the continuity equation. Fluid forced through the reduced area must accelerate, and the gain in kinetic energy comes at the expense of static pressure. Applying the two equations between an upstream section and the vena contracta ( the plane of minimum jet area just downstream of the plate) gives an expression in which discharge is proportional to the square root of the differential head.

This ideal expression cannot be used directly, as it considers a frictionless fluid and requires the area of the vena contracta, which cannot be measured. Real flow suffers viscous shear, turbulent dissipation and losses on sudden expansion, so the actual discharge always falls below the ideal value. Both effects are absorbed into the coefficient of orifice, C₀, the ratio of actual to theoretical discharge. Since it depends on the plate geometry, tapping positions and the condition of the orifice edge, C₀ must be determined experimentally rather than predicted. That determination is the purpose of this experiment, which also checks whether the linearised relationship (Q∝√∆H) holds and whether C₀ remains constant once the flow is fully turbulent.

# 2. Theory

## 2.1 Orifice Meter and its Principle of Operation

An orifice meter is a differential-pressure flow measuring device. It consists of a thin plate, pierced by a concentric circular hole, clamped between flanges in a pipeline. The plate obstructs the flow, and the fluid is forced to accelerate through the reduced area. Because the acceleration is achieved at the expense of static pressure, a measurable pressure difference develops across the plate. That pressure difference is the quantity actually measured; the flow rate is inferred from it by using Bernoulli’s equation. Pressure tappings are placed upstream of the plate and immediately downstream, and the difference is read on a manometer as a head of the flowing fluid, ΔH.

Figure 1: Diagram of an Orifice Meter [1]

## 2.2 Vena Contracta

As fluid passes through a restricting opening, the edges of the orifice plate deflect the streamlines inward along a curved path. The fluid's inertia carries this convergence beyond the plate, so the jet continues to narrow for a short distance after leaving the opening and attains a cross-sectional area smaller than that of the opening itself. This minimum area plane is the vena contracta; downstream of it the jet expands once more until it again fills the pipe

## 2.3  Bernoulli’s Equation

An expression linking the static pressure head difference to the volumetric flow rate is obtained by applying Bernoulli's theorem between two sections of the system: Point x, located upstream of the plate where the flow is uniform, and Point y, located at the vena contracta, the plane of minimum cross-sectional area immediately downstream of the orifice.

For an incompressible, frictionless fluid flowing steadily through a horizontal pipe, Bernoulli's equation is can be written as [2]:

Pxρg+vx22g+zx=Pyρg+vy22g+zy

Where,

Px,Py = Pressure at points x and y

vx,vy = Fluid velocities at points x and y

zx,zy = Elevation heads

ρ = Density of the fluid

g = Acceleration due to gravity

for a horizontal pipe,

zx=zy

Because the pipe is horizontal, the elevation terms cancel out. We can rearrange the equation to relate static pressure head difference with the difference of fluid velocities at points x and y:

∆H=vx2-vy22g

## 2.4 Continuity Equation

The continuity equation is the mathematical statement of mass conservation applied to a flowing fluid. Since mass can neither be created nor destroyed, the rate at which mass enters a given control volume must equal the rate at which it leaves.

If the flow is steady and incompressible, the continuity equation can be defined as:

A1vx=A0vy

Where,

A1 = Cross-sectional area of the pipe

Ao = Cross-sectional area of the orifice

Rearranging this to express the upstream velocity (vA) in terms of the orifice velocity (vB):

vx=vyAoA1

## 2.5 Differential Manometer Relation

The pressure difference across the orifice is measured with a U-tube differential manometer connected between the upstream and downstream tappings. Since both limbs are subject to the same column of flowing fluid above the manometer liquid, the difference in static pressure between the two sections is given by:

Px-Py=Rgρm-ρ

Where ,

R = manometer reading difference

ρm= density of the manometer liquid

ρ = density of the flowing fluid. Expressing this as a head of the flowing fluid gives:

ΔH=Px-Pyρg=Rρmρ-1
The manometer used in this experiment used CCl4 as manometric fluid and the fluid flow was water, so the differential manometer relation in this case can be written as:

∆H= RρCCl4-ρwρw

Where,

∆H = Pressure head difference

R = Manometer reading difference

ρCCl4 = Density of CCl4

ρw = Density of H2O

## 2.6 Derivation of the Governing Equation

To  use the orifice meter properly, we must first establish the theoretical formula for its flow rate. This model is derived by applying Bernoulli’s Principle and the Continuity Equation between two distinct locations in the system: Point x and Point y. Substituting the expression vx from continuity equation in the static pressure head difference equation leads to:

ΔH=vy2-vy2AoA122g

Factoring out vB2:

ΔH=vy21-AoA1 22g

Solving for the theoretical velocity at the orifice (vy):

vy=2gΔH1-AoA12

Since the cross-sectional area of a circular pipe is proportional to the square of its diameter (A1=π4D12 and Ao=π4Do2), the area ratio can be expressed in terms of the orifice diameter (Do) and the pipe diameter (D):

AoA1=Do2D12=DoD12

Substituting this into the velocity equation gives:

vy=2gΔH1-DoD14

Finally, the theoretical discharge (QTheo) is the product of the orifice area and the theoretical velocity at Point y:

QTheo=Aovy=Ao2gΔH1-DoD14

## 2.7 Coefficient of Orifice

The theoretical discharge equation derived above assumes an ideal, frictionless fluid. No real flow satisfies this condition, since some energy is always dissipated as the fluid passes through the meter. The discharge measured in practice (QExp) therefore falls consistently below the theoretical value, the shortfall arising from two effects: frictional and turbulent energy losses, and contraction of the jet to an area smaller than the machined orifice.

Both effects are accounted for by a single empirical factor, the coefficient of orifice (C₀), defined as the ratio of the measured discharge to the theoretical discharge

Co=QExpQTheo

Substituting QTheo,

Co=QExpAo2gΔH1-DoD14

In practice, QActual is determined physically during the experiment by using a flowmeter. If theoretical discharge is calculated using manometer readings (ΔH) and the equation derived earlier, Co can be directly determined.

## 2.8 Types of Orifice Plates

Orifice plates are manufactured in several geometries, the choice depending on the nature of the fluid being metered. The sharp-edged concentric plate is the most common, and is suited to clean fluids in turbulent flow. The eccentric plate carries its opening off-centre, which prevents the accumulation of sediment and makes it appropriate for fluids carrying suspended solids. The segmental plate, whose opening is a circular segment, serves a similar purpose where the stream contains solid particles or entrained gas bubbles. The conical plate has a tapered inlet that admits the fluid more gradually, giving a smoother entry and a smaller energy loss than the square-edged form. [3]

Figure 2:Different types of orifice plates based on geometry [3]

## 2.9 Reynold’s Number

The Reynolds number is a dimensionless quantity that represents the ratio of inertial forces (which propel the fluid forward) to viscous forces (which resist flow and cause friction) within the system. For pipe flow, it is calculated using the conditions in the main pipe upstream of the orifice (Point A):

Re=ρvADμ

Where:

- ρ = Density of the fluid

- vA = Mean fluid velocity in the upstream pipe

- D = Internal diameter of the main pipe

- μ = Dynamic viscosity of the fluid

# 3. Materials and Methods

## 3.1 Apparatus

- Orifice plate

- Differential manometer

- Centrifugal pump

- Reservoir

- Flowmeter

- Stopwatch

- Thermometer

- Scale

## 3.2 Experimental Setup

Manometer fluid

Scale

Flowmeter

Manometer

Pressure Control

Valve

Bypass Valve

Supply Valve

Entrance valve

Orifice Plate

Main Valve

Pump

Figure 2: Schematic Diagram of the Orifice Meter Apparatus

The apparatus consists of a closed water circulation system arranged for the calibration of an orifice meter. Water is drawn from the reservoir by a centrifugal pump, and the delivery is regulated by a network of valves: a main valve, an entrance valve and a bypass valve ; which together allow the flow to be set and held steady before it enters the horizontal test section. The orifice plate is mounted at the mid-length of this section, where it provides the restriction that produces the pressure drop. Flexible tubing connects tapping points immediately upstream and downstream of the plate to a U-tube manometer charged with carbon tetrachloride, from which the differential head is read. Beyond the plate the water passes through a pressure control valve, used to vary the flow rate between runs, and then through a flowmeter, before being discharged through the outlet pipe and returned to the reservoir.

## 3.3 Experimental Procedure

Standard safety protocols were observed before starting. The centrifugal pump was switched on to fill the reservoir and circulate water through the system, and the inlet and bypass valves were adjusted to establish a steady flow. One member of the group was assigned to regulate the supply valve throughout, keeping the reservoir level constant and preventing overflow. The tapping connections on either side of the orifice plate were checked for leaks, and the initial manometer levels were noted as a baseline.

The pressure control valve was then set to give the first test flow rate, and the system was left to reach steady state before any reading was taken. The volumetric flow rate was obtained by timing the flowmeter over a measured interval with a stopwatch, while the levels in both limbs of the manometer were recorded simultaneously, care being taken to minimise reading error caused by pump vibration. The control valve was reopened in stages to raise the flow rate, and the procedure was repeated until eight sets of readings had been collected.

Finally, the room and water temperatures were measured with a thermometer. These were needed to fix the density and viscosity of water, which together with the flow and pressure measurements were used to compute the theoretical parameters and construct the calibration curves.

# 4. Results and Discussions

## 4.1 Calibration Data of the Orifice Meter

Table 1: Data for Volumetric Flow Rate, Pressure Head Difference, Reynold's Number and Coefficient of Orifice

| Observation No. | Volumetric Flow Rate, Q×104 (m3/s) | Pressure Head Difference, ΔH×102 (m H2O) | Square Root of Pressure Head Difference, ∆H (m H2O) | Reynold’s Number, Re | Coefficient of Orifice, Co |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.81 | 4.19 | 0.2047 | 11300 | 0.8067 |
| 2 | 4.20 | 5.18 | 0.2276 | 12454 | 0.7998 |
| 3 | 5.03 | 6.57 | 0.2563 | 14839 | 0.8505 |
| 4 | 5.19 | 7.97 | 0.2823 | 15407 | 0.7967 |
| 5 | 5.78 | 9.19 | 0.3032 | 17140 | 0.8263 |
| 6 | 6.48 | 11.58 | 0.3403 | 19259 | 0.8253 |
| 7 | 6.81 | 12.74 | 0.3569 | 20222 | 0.8269 |
| 8 | 6.99 | 13.61 | 0.3689 | 20735 | 0.8212 |

The calibration data from the experiment showed in Table 1 indicates that raising the experimental volumetric flow rate from 3.81×10-4 m³/s to 6.99×10-4 m³/s produced a steady increase in the pressure head difference (ΔH), from 4.19×10-2 m H₂O to 13.61×10-2 m H₂O. This trend is consistent with the principles of fluid dynamics outlined in the theoretical model. The table also reports the square root of the pressure head difference (ΔH), which is required for plotting the linear calibration relationship .

The Reynolds numbers (Re) calculated for the test runs fell between 11300 and 20735, confirming that the fluid flow remained entirely within the turbulent  regime (Re>4000)  during all eight observations. Turbulent conditions stabilise the jet at the vena contracta and keep the discharge coefficient nearly constant . Finally, the Coefficient of Orifice meter (Co) is measured for each trial, showing values that ranged from  0.7967 to 0.8269, satisfying theoretical condition C₀ < 1.0 which arises from frictional losses, turbulence, and contraction of the jet . Even though the coefficient value of a square edged orifice meter typically ranges between 0.6 and 0.62 under turbulent flow conditions the observed deviations are caused by mainly  experimental factors, notably minor wear or rounding along the upstream edge of the orifice plate, which weakened streamline contraction and increased the effective area of the vena contracta. Taken as a whole, the results agree closely with theoretical expectations, confirming that the orifice meter was successfully calibrated for practical flow measurement.

## 4.2 Calibration Curves of the Orifice Meter

Figure 3: Plot of Pressure head difference vs Volumetric flowrate

Plotting differential pressure head (ΔH) against volumetric flow (Q) rate produced a curve that steepens with increasing discharge .This behaviour follows directly from Bernoulli's principle. and the conservation of energy .When fluid is forced through the reduced area of the orifice, it must speed up to satisfy continuity, and by the conservation of energy that gain in kinetic energy can only come from the static pressure head loss. The velocity at the throat scales linearly with Q, so the head lost across the orifice scales with Q² which makes an ideal parabola. What the measured points show is that same quadratic behaviour: each successive increment in flow rate costs more pressure head than the one before it, and the fitted curve reproduces the predicted profile within the scatter of the data. Not precisely using the stopwatch when focusing on the rotation of the dial of the flowmeter is responsible for minor deviations from ideal parabolic behavior .

Figure 4: Plot of Square root of Pressure head difference vs Volumetric flowrate

To confirm the quadratic dependence in a more readily verifiable form, the data were replotted as square root of the pressure head difference (ΔH) vs actual volumetric flow rate (Q) in Figure 4. The transformation produced a straight-line calibration fit. This linear response follows from the governing orifice flow equation (Appendix D), obtained by combining Bernoulli's principle with the continuity equation, which establishes that volumetric flow rate is directly proportional to the square root of the differential pressure head (Q ∝ΔH).

Figure 5: Plot of Coefficient of orifice meter vs Pressure head difference

Figure 5 shows the relationship between the calculated coefficient of orifice (C₀) and the pressure head difference (ΔH). In theory, once the flow is fully turbulent, C₀ should stay constant with head, since C₀ is the ratio of actual to theoretical discharge and both scale with  √ΔH. The fitted trendline is indeed almost flat across the tested range of 0.042 m H₂O to 0.136 m H₂O, sloping very slightly upward, with values between 0.797 and 0.827. The small rise can be attributed to the higher velocities and Reynolds numbers at larger heads, where viscous effects become less significant and the actual flow approaches the ideal value more closely. The individual points, however, scatter noticeably about the line, most clearly the elevated value near ΔH = 0.066 m H₂O. This scatter is largely a reading problem rather than a real physical trend. Carbon tetrachloride is much lighter than mercury, so the manometer levels responded readily to vibration from the nearby centrifugal motor and were difficult to read steadily. Minor fluctuations in the pump supply and small air bubbles trapped in the manometer tubing would have added further uncertainty. Taken together, these practical limitations account for the variation seen in the calculated coefficients.

Figure 6: Plot of Coefficient of orifice meter vs Square root of pressure head difference

Figure 6 presents the same coefficient data replotted against the square root of the pressure head difference (ΔH). Since discharge varies directly with (ΔH), this axis is effectively a velocity scale, so the figure shows how the calibration coefficient responds as flow velocity is raised in every steps rather than in the compressed manner of the previous plot. Over the range 0.204 to 0.369 √𝑚 H2O, the coefficient again holds close to a constant value, drifting upward only slightly from about 0.80 at the lowest velocity to roughly 0.82 at the highest. The individual points remain scattered about the fitted line, with the reading near ΔH = 0.256 standing well above its neighbours, so the same experimental limitations discussed earlier evidently persist across the full velocity range and are not confined to the low-flow readings alone. Even so, the near-constancy of C₀ is the outcome the calibration set out to establish. A single averaged coefficient, taken from this fit, can be inserted into the orifice equation to convert any subsequent manometer reading into a flow rate, which is the practical purpose of calibrating the meter.

Figure 4: Plot of Coefficient of orifice meter vs Reynold's number

Figure 7 completes the graphical analysis by plotting the coefficient of orifice (C₀) against the Reynolds number (Re). The tested range of 11,300 to 20,735 lies well inside the fully turbulent regime, where inertial forces dominate viscous ones and C₀ should be a constant value independent of flow rate. The measured points broadly bear this out, though the fitted line rises marginally rather than running perfectly level, and the scatter noted in the earlier plots is again present.

More significant than the trend is the magnitude of the coefficient itself. The average value found here, about 0.81, is much higher than the 0.60 - 0.65 usually obtained from a sharp-edged plate. The most likely explanation is the condition of the upstream edge. A square, sharp edge forces the fluid to turn abruptly on entering the bore, which separates the flow cleanly and contracts the jet tightly. Wear or rounding of that edge because of using for long time softens the turn, weakens the contraction, and enlarges the effective area at the vena contracta, so more fluid passes for a given pressure drop and the calculated C₀ rises accordingly.

# 5. Conclusion

The orifice meter was calibrated by measuring the flow rate directly, using timed volumetric collection and comparing it with the ideal flow rate predicted by Bernoulli's principle and the continuity equation. Eight readings were taken, covering Reynold’s numbers from 11300 to 20735, so the flow stayed fully turbulent for the whole experiment. This is the range in which the coefficient of orifice should not depend on flow rate.

The measurements supported the theoretical treatment of the meter. The head difference grew in a parabolic manner as the discharge increased, and the same data formed a straight line when plotted against ΔH, which confirms the expected relationship (Q ∝ΔH). The coefficient itself stayed close to constant across the range, rising only very slightly at the higher heads.

The measured discharge was smaller than the ideal value in every run, which is what should happen: the ideal equation ignores the narrowing of the jet after the plate and the energy lost to friction, eddies and sudden expansion. The coefficient came out higher than the 0.60-0.65 usually found for a sharp-edged plate, and the reason is the worn upstream edge discussed. This is a fault of the apparatus rather than the method, so taking more readings would not remove it.

The points also scattered around the fitted lines. Most of this came from the manometer, which could only be read to ±0.5 cm and whose fluid kept moving because of vibration from the centrifugal motor. The effect was worst at low flow rates, where ΔH is small to begin with. Stopwatch timing by hand, air bubbles in the tubing, and small changes in the pump supply added a little more.

Even with these problems, the calibration curves describe how this particular meter behaves, and the average coefficient taken from them can be used to work out unknown flow rates from future manometer readings. The aim of the experiment was therefore achieved, though the coefficient found here belongs to this worn plate and should not be treated as a general value for orifice meters.

# References

[1] "What is Orifice Meter? Working Principle, Construction and Diagram," Electrical and Electronics Blog, Dec. 29, 2024. [Online]. Available: https://howelectrical.com/orifice-meter/. [Accessed: Jul. 23, 2026].

[2] Y. A. Çengel and J. M. Cimbala, Fluid Mechanics: Fundamentals and Applications, 4th ed. New York, NY, USA: McGraw-Hill Education, 2018.

[3] https://www.linkedin.com/pulse/what-orific-plate-mary-fitting-flange-kong

[4] The Engineering ToolBox, "Steel Pipes - Schedule 40," 2003. [Online]. Available: https://www.engineeringtoolbox.com/steel-pipes-dimensions-d_43.html. [Accessed: Jul. 23, 2026].

# List of Symbols

| Symbol | Meaning | Unit (SI) |
| --- | --- | --- |
| A | Cross-sectional area of the pipe | m² |
| Ao | Cross-sectional area of the orifice | m² |
| Co | Coefficient of Orifice | Dimensionless |
| D | Internal diameter of the main pipe | m |
| Do | Orifice diameter | m |
| g | Acceleration due to gravity | m/s² |
| PA, PB | Static pressure at points A and B | Pa |
| QExperimental | Experimental volumetric flow rate | m³/s |
| QTheoretical | Theoretical volumetric flow rate | m³/s |
| R | Manometer reading difference | m |
| Re | Reynolds Number | Dimensionless |
| t | Time interval | s |
| V | Volume of water | m³ |
| v | Average fluid velocity | m/s |
| vA, vB | Fluid velocities at points A (upstream) and B (vena contracta) | m/s |
| zA, zB | Elevation heads | m |
| ΔH | Pressure head difference | m |
| μw | Dynamic viscosity of water | Pa⋅s |
| ρ | Density | kg/m³ |
| ρw | Density of water | kg/m³ |
| ρCCl4 | Density of Carbon Tetrachloride (manometric fluid) | kg/m³ |

# Appendices

## Appendix A

### Observed Data

Room Temperature = 29.5℃

Water Temperature = 29℃

Schedule Number = 40

Pipe Diameter, D = 2 inch = 0.0525 m [4]

Orifice Diameter, Do = 1 inch = 0.0254 m

Table 2: Observed Data for Volume of Water, Time Interval and Manometer Reading

| Observation No. | Volume of Water (L) | Time Interval, t (s) | Manometer Reading |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Left (cm) | Right (cm) | Difference, R (cm) |
| 1 | 5.0 | 13.12 | 32.8 | 25.6 | 7.2 |
| 2 |  | 11.90 | 33.6 | 24.8 | 8.9 |
| 3 |  | 9.94 | 35 | 23.7 | 11.3 |
| 4 |  | 9.63 | 36.5 | 22.8 | 13.7 |
| 5 |  | 8.65 | 38.5 | 21.7 | 15.8 |
| 6 |  | 7.71 | 39.5 | 19.6 | 19.9 |
| 7 |  | 7.34 | 40.7 | 18.8 | 21.9 |
| 8 |  | 7.15 | 41.2 | 17.8 | 23.4 |

## Appendix B

### Calculated Data

Density of water at 29℃, ρw =995.94 kg/m3 [1]

Viscosity of water at 29℃, μw = 8.145×10-4 Pa⋅s [1]

Density of CCl4 at 29.5℃, ρCCl4=1575 kg/m3 [1]

Area of Orifice, Ao = 5.067×10-4 m2

Area of Pipe, A = 2.165×10-3 m2

Table 3: Table for Calculated Data of Volumetric Flow Rate, Velocity and Reynold's Number

| Observation No. | Volume of Water, V×103 (m3) | Time, t (s) | Volumetric Flow Rate, Q×104 (m3/s) | Velocity, v (m/s) | Reynolds number |
| --- | --- | --- | --- | --- | --- |
| 1 | 5.0 | 13.12 | 3.81 | 0.176 | 11300 |
| 2 |  | 11.90 | 4.20 | 0.194 | 12454 |
| 3 |  | 9.94 | 5.03 | 0.232 | 14839 |
| 4 |  | 9.63 | 5.19 | 0.240 | 15407 |
| 5 |  | 8.65 | 5.78 | 0.267 | 17140 |
| 6 |  | 7.71 | 6.49 | 0.300 | 19259 |
| 7 |  | 7.34 | 6.81 | 0.315 | 20222 |
| 8 |  | 7.15 | 6.99 | 0.323 | 20735 |

Table 4: Table for Calculated Data of Pressure head difference, Square root of pressure head difference and Orifice coefficient

| Observation No. | Pressure head difference, ∆H×102 (m H2O) | Square root of pressure head difference ∆H (m H2O) | Volumetric flow rate , Q×104 (m3/s) | Orifice coefficient, Co |
| --- | --- | --- | --- | --- |
| 1 | 4.19 | 0.2047 | 5.00 | 0.805 |
| 2 | 5.18 | 0.2276 | 5.56 | 0.798 |
| 3 | 6.57 | 0.2563 | 5.81 | 0.848 |
| 4 | 7.97 | 0.2823 | 6.25 | 0.795 |
| 5 | 9.19 | 0.3032 | 6.45 | 0.9799 |
| 6 | 11.58 | 0.3403 | 7.51 | 0.824 |
| 7 | 12.74 | 0.3569 | 8.25 | 0.825 |
| 8 | 13.61 | 0.3689 | 8.94 | 0.819 |

## Appendix C

### Sample Calculation

For Observation No. 4,

Measured with the flow meter, volume of water, V = 5.0 ×10-3 m3

Time interval, t = 9.63 s

Volumetric flow rate of water, Q = Vt = 5.19 ×10-4  m3/s

Pipe Diameter (Schedule No. 40), D = 2.067inch = 0.0525 m

Area of Pipe, A = π × D24  = 2.165 ×10-3 m2

Average velocity of water, v = QA = 0.240 m/s

Manometer reading,

Left reading = 36.5 cm CCl4

Right reading = 22.8 cm CCl4

Manometer reading difference, R = (36.5 – 22.8) cm CCl4 = 13.7 cm CCl4

= 13.7×10-2 m CCl4

Orifice Diameter, Do = 1 inch = 0.0254 m

Area of Orifice, Ao = π × D024  = 5.067 ×10-4 m2

Pressure Head difference, ∆H = R ×ρCCl4ρw -1

= 13.7×10-2 ×1575995.94-1 m H2O

= 0.0797 m H2O

Square Root of Pressure Head Difference, ΔH = 0.28 √m H2O

Reynolds Number, Re = Dvρw μw = 0.0525× 0.240×995.94 8.145×10-4 = 15397

Coefficient of discharge, Co= QA02g∆H1-(DoD)4 = 5.19 ×10-4  5.067×10-4×√2×9.8×0.07971-(0.02540.0525)4 = 0.7967

Marking scheme: Long report on ‘Experiment 4: Calibration of Orifice Meter’

Name: Tajrian Elmi Rashid

Student number: 2302034

| Assessment Component | Weight (%) | Obtained Marks |
| --- | --- | --- |
| Summary | 10% |  |
| Experimental Set up | 15% |  |
| Result and Discussion and Conclusion | 25% |  |
|  | Subtotal (50%) |  |
| Introduction and Theory | 15% |  |
| Reference and Nomenclature | 05% |  |
| Quality of Tables | 10% |  |
| Quality of Figures | 10% |  |
| Overall Presentation | 10% |  |
| Total | 100% |  |