Bangladesh University of Engineering & Technology

DEPARTMENT OF CHEMICAL ENGINEERING

Course No: ChE-206

Course Title: Chemical Engineering Laboratory I

A Long Report on

Experiment No 04: CALIBRATION OF ORIFICE METER

Date of Performance: 22/07/2026

Date of submission: 19/08/2026

Submitted to:

Mahmud Hayet Rudro 
Lecturer, Department of Chemical Engineering 
Bangladesh University of Engineering and Technology, Dhaka-1000

Submitted by:

Shahriar Hossain Shohan

ID:2302042 
Lab Group: 03  
Section: A2; Level/Term: 2/2

Lab Partners:

Faizur Rahman Shawon 		Student ID: 2302041

Nafisa Tabassum Rodela		Student ID: 2302043

Md. Touhid Hossen 		Student ID: 2302044

Taef Jalal Patoary			Student ID: 2302045

# Statement of academic Integrity:

By submitting this formal report on the “Calibration of Orifice Meter”, I certify that all contained work, data analyses, calculations, and written interpretations are entirely my own. I have not copied the work of any other student, past or present. All external resources and references have been fully acknowledged according to institutional academic standards.

Name: Shahriar Hossain Shohan	
Student ID: 2302042

Date: 19/08/2026

# Summary:

The experiment “Calibration of Orifice Meter” was performed to observe the operation and working principle of an orifice meter, construct calibration curves correlating volumetric flow rate with differential pressure head, and evaluate the relationship between the coefficient of discharge and the pipe Reynolds number or pressure head. The experimental setup consisted of an orifice plate with a 1-inch diameter orifice coaxially fitted inside a 2-inch Schedule 40 pipeline through which water was passed, with a U-tube differential manometer containing carbon tetrachloride (CCl4) used as the manometric fluid to measure pressure drops across the orifice plate. The orifice meter relies on the Bernoulli principle and continuity equation, where restricting the flow channel increases local fluid velocity and causes a corresponding drop in static pressure. Since the real fluids undergo energy losses from internal friction, turbulence, and flow separation, the theoretical volumetric flow rate must be adjusted using an empirical correction factor called the Coefficient of Discharge (Cd). In this experiment, eight different flows were observed, where volumetric flow rates varied from 4.69×10⁻⁴ m³/s to 7.80×10⁻⁴ m³/s, corresponding to Reynolds numbers ranging from 13866.43 to 23110.72, which confirms that the fluid flow was fully turbulent across all observations. Theoretically, the differential pressure head should increase quadratically with volumetric flow rate, while the square root of the pressure head should exhibit a strict linear relationship with flow rate. The experimental findings followed these theoretical predictions closely, with differential pressure head increasing from 4.38 × 10⁻2 m H₂O to 17.53×10⁻2 m H₂O and displaying a clear linear trend when plotting the square root of pressure head against volumetric flow rate. The experimental orifice coefficient values ranged from a maximum of 0.97 at the lowest flow rate to a minimum of 0.73, before stabilizing between 0.79 and 0.81 with an average of approximately 0.80 for the remaining observations, which aligns closely with expected turbulent flow characteristics despite slight initial low-velocity anomalies.

# Acknowledgement:

I am sincerely grateful to be a part of this experiment titled “Calibration of Orifice Meter.” This experiment was performed under a sessional course named Chemical Engineering Laboratory I, which aims to familiarize students with core fluid mechanics principles through a hands-on approach and demonstrate how textbook theories apply to real-world engineering.

The primary objective of the experiment is to establish the relationship between differential pressure head and volumetric flow rate and to determine the discharge coefficient through calibration measures. This experiment was conducted at the Fluid Mechanics Lab of the Department of Chemical Engineering inside the Old Academic Building at Bangladesh University of Engineering and Technology (BUET), Dhaka-1000. I am very thankful to the Department of Chemical Engineering, BUET, for providing the necessary laboratory facilities and resources required to perform this experiment.

I would like to express my heartfelt gratitude to my respected teacher, Mahmud Hayet Rudro, Lecturer, Department of Chemical Engineering, BUET, for explaining the theoretical background, guiding us through the apparatus setup, and offering continuous support during the experiment and report preparation. Special thanks go to the laboratory instructors and technical staff for providing necessary assistance and ensuring proper setup operation. I would also like to acknowledge my group members and classmates for their coordinated efforts and enthusiasm in conducting this experiment successfully. Finally, I sincerely appreciate all the teachers, staff members, and benefactors who made this laboratory session possible.

Name: Shahriar Hossain Shohan	
Student ID: 2302042
Department of Chemical Engineering
Bangladesh University of Engineering and Technology

# Table of Contents

Statement of academic Integrity:	i

Summary:	ii

Acknowledgement:	iii

Table of Contents	iv

List of Figures	v

List of Tables	v

1. Introduction	1

2. Theory	2

2.1 Orifice Meter and Vena Contracta	2

2.2 Bernoulli’s Equation	3

2.3 Continuity Equation	4

2.4 Differential Manometer Relation	5

2.5 Theoretical Discharge Equation in an Orifice Meter	5

2.6 Coefficient of Discharge	6

2.7 Reynolds Number	6

2.8 Calibration of Orifice Meter	7

3. Materials and Methods	8

3.1 Apparatus	8

3.2 Experimental Setup	8

3.3 Experimental Procedure	9

4. Result and Discussion	10

5. Conclusion	16

References	17

List of Symbols	18

Appendices	19

Appendix A	19

Observed Data	19

Appendix B	20

Calculated Data	20

Appendix C	21

Sample Calculation	21

# List of Figures

Figure 1: Diagram of an orifice meter	2

Figure 2: Schematic Diagram of the Orifice Meter Apparatus	8

Figure 3: Plot of Pressure Head Difference vs Volumetric Flowrate	11

Figure 4: Plot of Root of Pressure Head Difference vs Volumetric Flowrate	12

Figure 5: Plot of Coefficient of Orifice Meter vs Pressure Head Difference	13

Figure 6: Plot of Coefficient of Orifice Meter vs Root of Pressure Head Difference	14

Figure 7: Plot of Coefficient of Orifice Meter vs Reynolds Number	15

# List of Tables

Table 1: Data for Volumetric Flow Rate, Pressure Head Difference, Reynolds Number and Coefficient of Orifice	10

Table 2: Observed data table for volume of water, time interval, manometer reading	19

Table 3: Table for Calculated Data of Volumetric Flow Rate, Velocity and Reynolds Number	20

Table 4: Table for Calculated Data of Pressure Head Difference, Square Root of Pressure Head Difference and Orifice Coefficient	21

# 1. Introduction

Accurate measurement and control of fluid flow rates is one of the fundamental requirements in chemical process plants, refineries, and water distribution networks. Flow meters enable engineers to monitor process conditions, ensure product quality, optimize energy efficiency, and maintain operational safety. Among the various types of flow measurement devices available in fluid dynamics, the “Orifice Meter” is one of the most common differential pressure flow meters due to its structural simplicity, low manufacturing cost, ease of installation, and lack of moving parts.

An orifice meter operates by introducing a sudden restriction, a thin metal plate with a precise concentric circular hole, into a pipeline. As fluid approaches and passes through this restricted orifice opening, the stream accelerates, reaching its maximum velocity and minimum cross-sectional area at a region just downstream known as the ‘Vena Contracta’. According to Bernoulli’s principle [1], this acceleration converts static pressure head into kinetic energy, generating a measurable static pressure drop across the orifice plate. By connecting a U-tube differential manometer across the orifice plate, this pressure head difference (ΔH) can be recorded and correlated with the actual volumetric flow rate (Q).

However, in real fluid flow applications, energy losses inevitably occur due to internal fluid friction, boundary layer separation, eddy formation, and jet contraction at the orifice opening. Consequently, theoretical flow rate equations derived from ideal fluid mechanics overestimate actual flow rates. To reconcile these practical energy losses, an empirical correction factor, the coefficient of discharge (Cd), is used to calibrate the meter. Coefficient of discharge (Cd) is not a universal constant. It varies with shape, size, pipe diameter and such other factors. So, each orifice meter has a different coefficient of discharge (Cd) according to the parameters. Every orifice meter must be calibrated according to its own parameters.

# 2. Theory

## 2.1 Orifice Meter and Vena Contracta

An orifice meter is a type of differential pressure flow meter used to measure the volumetric flow rate of a fluid passing through a closed pipe or conduit. It consists of a thin metal plate with a precise, sharp edged circular opening (the orifice) mounted coaxially inside a pipeline. When fluid flows through the pipe, the sudden reduction in cross-sectional area forces the fluid stream to accelerate.

When fluid approaches the sharp edge of an orifice opening, the fluid streamlines cannot execute an abrupt 90∘ turn. Due to fluid inertia, the converging streamlines continue to contract even after passing physically through the orifice hole. As a result, the narrowest point of the fluid jet does not occur right at the plate, but slightly downstream. That point is called ‘Vena Contracta’. The ‘Vena Contracta’ is the region in a fluid stream downstream of a constriction where the cross-sectional area of the fluid jet reaches its absolute minimum, and fluid velocity reaches its absolute maximum. The distance of vena contracta depends on the ratio of the orifice diameter and pipe diameter, represented by the diameter ratio β=DoD. For smaller β, the vena contracta forms further downstream from the plate.

Figure 1: Diagram of an orifice meter [2]

## 2.2 Bernoulli’s Equation

Bernoulli’s Principle [1] states that for an incompressible, frictionless fluid undergoing steady, streamlined flow, the total mechanical energy of the fluid remains constant along any given streamline. In simple terms, as the velocity (v) of a fluid increases, its static pressure (P) or potential energy (z) must decrease simultaneously to keep the total energy constant.

Assuming an ideal, incompressible, and frictionless fluid flowing through a horizontal pipeline, applying Bernoulli’s equation between an upstream point (1) and the orifice/vena contracta location (2) yields:

P1ρg+v122g+z1=P2ρg+v222g+z2

Where:

P1,P2 = Static Pressure at points 1 and 2 (Pa)

v1,v2 = Fluid velocities at points 1 and 2 (m/s)

z1,z2 = Elevation heads (m)

ρ = Density of the fluid (kg/m3)

g = Acceleration due to gravity (m/s2)

For a horizontal pipe, z1=z2. So, the equation can be rearranged to relate the static pressure head difference of fluid with velocities at point 1 and 2.

ΔH=P1-P2ρg=v22-v122g

## 2.3 Continuity Equation

The Continuity equation [1] states that for a continuous flow of fluid through a system, mass cannot be created or destroyed. Therefore, for any control volume (such as a pipe or channel), the rate at which mass enters the system must equal the rate at which mass exits the system plus any accumulation of mass within the system:

Mass Flow Ratein m1=Mass Flow Rateout m2

Or,		 ρ1A1v1=ρ2A2v2

Where:

ρ1,ρ2 = Fluid densities at sections 1 and 2 (kg/m3)

A1,A2 = Cross-sectional areas at sections 1 and 2 (m2)

v1,v2 = Average fluid velocities at sections 1 and 2 (m/s)

For incompressible fluids (where density remains constant, ρ1=ρ2=ρ), the density terms cancel out. The equation simplifies to the conservation of volumetric flow rate (Q) for an orifice meter:

Q=Av1=A0v2=Constant

Or, v1=v2A0A=v2D0D2

Where:

A = Cross sectional area of the pipe (m2)

Ao = Cross sectional area of the orifice (m2)

D = Diameter of the pipe (m)

Do = Diameter of the orifice (m)

In an orifice meter, the flow area reduces at the orifice opening, and according to the continuity

equation, the fluid velocity increases at this section. This increase in velocity leads to a

corresponding decrease in static pressure. Thus, the continuity equation explains the rise in

velocity that contributes to the pressure difference used for flow measurement.

## 2.4 Differential Manometer Relation

A differential manometer relation is used to calculate the pressure difference between two points in a fluid system with respect to another fluid other than the manometric fluid. The manometer used in this experiment used CCl4 as manometric fluid and the fluid flow was water, so the differential manometer relation in this case is expressed as:

∆H= RmρCCl4-ρwρw

Where:

∆H = Pressure head difference (m H2O)

Rm= Manometer reading (m CCl4)

ρCCl4 = Density of CCl4

ρw = Density of H2O

## 2.5 Theoretical Discharge Equation in an Orifice Meter

The Theoretical Discharge Equation for an orifice meter is derived by applying Bernoulli’s Principle and the Continuity Equation between an upstream point in the pipe (Point 1) and the orifice opening / vena contracta (Point 2). By combining the continuity equation with Bernoulli’s equation:

ΔH=v22-v22AoA22g=v221-AoA22g

Or,     v2=2gΔH1-AoA2 =2gΔH1-DoD4

Finally, the theoretical discharge (Qth) is the product of the orifice area and the theoretical velocity at Point 2:

Qth=Aov2=Ao2gΔH1-DoD4

## 2.6 Coefficient of Discharge

The theoretical equation of discharge assumes a perfect, frictionless fluid flow. It doesn’t account for any energy losses. In real fluid applications, energy losses occur due to fluid friction, turbulence, and jet contraction at the vena contracta. To convert theoretical discharge into actual discharge, a Coefficient of Discharge (Cd) is introduced. Cd is the ratio of the actual flow measured and the theoretical flow rate:

Cd=QQth

Hence, 
Cd=QAo2gΔH1-DoD4

In this experiment, a flow meter was used to measure Q and manometer reading (ΔH) was used to calculate Qth. Cd is further measured using the equation derived earlier.

## 2.7 Reynolds Number

The Reynolds Number (Re) is a fundamental dimensionless quantity in fluid mechanics used to predict fluid flow patterns in different situations. It measures the ratio of inertial forces to viscous forces within a flowing fluid. For pipe flow, it is calculated using the conditions in the main pipe upstream of the orifice (Point 1):

Re=ρv1Dμ

Where:

- ρ = Density of the fluid (kg/m3)

- v1 = Mean fluid velocity in the upstream pipe (m/s)

- D = Internal diameter of the main pipe (m)

- μ = Dynamic viscosity of the fluid (Pa.s)

## 2.8 Calibration of Orifice Meter

Orifice meter calibration connects theoretical fluid equations with actual device performance by accounting for practical energy losses, fluid properties, and geometric factors reflected in the discharge coefficient (Cd). To ensure measurement accuracy, the meter is tested under controlled steady-flow conditions. Consequently, an uncalibrated meter yields inaccurate flow measurements and requires testing under controlled conditions to establish a baseline.

During calibration, fluid flows through the setup at several steady flow rates. At each setting, the actual volumetric flow rate (Q) is directly measured using a standard calibrated flow meter, while the differential pressure head (ΔH) across the orifice plate is recorded from a manometer. The discharge coefficient (Cd) is computed for each run and plotted against parameters such as the pipe Reynolds number (Re), ΔH, and ΔH to construct calibration curves. Once calibrated, the device can reliably measure unknown flow rates in practical engineering applications.

# 3. Materials and Methods

## 3.1 Apparatus

- Orifice plate

- Differential manometer

- Centrifugal pump

- Reservoir

- Flowmeter

- Stopwatch

- Thermometer

- Scale

## 3.2 Experimental Setup

Figure 2: Schematic Diagram of the Orifice Meter Apparatus

The experimental setup for the orifice meter calibration consisted of a closed-loop fluid flow circuit designed to measure differential pressure and actual volumetric discharge under steady-state conditions. Water was drawn from a reservoir and continuously circulated through a horizontal pipe network by a centrifugal pump. The flow was mainly controlled by the main valve along with other valves that ensure a steady flow. A bypass was used to redirect excess fluid to the reservoir so that there could be no sudden change in the velocity of the fluid in any portion of the pipe other than the orifice plate. Installed within the pipeline was an orifice plate, a thin metal plate with a precise central restriction of 1 inch diameter, clamped securely between two pipe flanges to induce a localized pressure drop as fluid passes through. Upstream and downstream pressure tapings around the restriction were connected by flexible tubes to a U-tube differential manometer filled with CCl4 to directly measure the differential head (ΔH) caused by the restriction. Downstream of the test section, a flow control valve was used to adjust and regulate the flow rate and a flow meter is connected to measure the flow rate.

## 3.3 Experimental Procedure

At the beginning of the experiment, the reservoir was filled with water before starting the centrifugal pump. To carry out the experiment, the main inlet valve was kept fully open while the water flow rate through the horizontal pipeline was incrementally adjusted and regulated using the outlet flow control valve. The water level of the reservoir was maintained at the same level to prevent overflow and to prevent any air bubbles from entering the system. Before recording data, the system was allowed to stabilize at each flow setting to ensure steady-state conditions, and air bubbles were purged from the connecting tubes of the U-tube differential manometer. At each of the eight (8) selected flow regimes, the differential manometric fluid levels (using CCl4) were recorded across the upstream and downstream pressure taps of the orifice plate to determine the corresponding pressure head loss (ΔH). At the same time, the actual volumetric flow rate (Q) was determined using a flow meter by measuring the time required to discharge 5 L of water with a stopwatch. Finally, both the water temperature and room temperature were measured and recorded to determine fluid properties such as density and dynamic viscosity.

# 4. Result and Discussion

Table 1: Data for Volumetric Flow Rate, Pressure Head Difference, Reynolds Number and Coefficient of Orifice

| No. of Observation | Volumetric Flow Rate, Q×104 (m3/s) | Pressure Head Difference, ΔH×102 (m H2O) | Reynolds Number, Re | Orifice Coefficient (Cd) |
| --- | --- | --- | --- | --- |
| 1 | 4.69 | 4.38 | 13866.43 | 0.960 |
| 2 | 5.05 | 8.95 | 14957.77 | 0.730 |
| 3 | 6.03 | 10.63 | 17846.61 | 0.800 |
| 4 | 6.64 | 12.85 | 19644.11 | 0.801 |
| 5 | 6.90 | 13.93 | 20414.47 | 0.799 |
| 6 | 7.14 | 15.01 | 21188.83 | 0.797 |
| 7 | 7.44 | 16.81 | 22019.38 | 0.785 |
| 8 | 7.80 | 17.53 | 23110.72 | 0.805 |

Table 1 summarizes the experimental results obtained during the calibration of the orifice meter for different flow rates. It is observed that the pressure head difference (ΔH) exhibited a rise from 4.38×10-2 m H2O to 17.53×10-2 m H2O corresponding to the volumetric flow rate ranging from 4.69×10-4 m3/s to 7.80×10-4 m3/s, confirming the fundamental theoretical relation where differential pressure head is proportional to the square of the flow rate. Across all tested flow regimes where Reynolds numbers varied from 13866.43 to 23110.72, the discharge coefficient (Cd) ranged between 0.73 and 0.96, maintaining an average value of approximately 0.8. At lower Reynolds numbers, minor variations in Cd were observed. However, as the flow entered the fully turbulent regime inertial forces dominated, causing the position of the vena contracta to stabilize and Cd to become virtually independent of the Reynolds number.

Figure 3: Plot of Pressure Head Difference vs Volumetric Flowrate

The graph illustrates a non-linear, upward curving parabolic relationship between the volumetric flow rate (Q) and the differential pressure head (ΔH), which directly aligns with fundamental fluid mechanics principles. According to Bernoulli's equation and the orifice discharge equation, the volumetric flow rate is proportional to the square root of the differential head (Q∝ΔH), meaning that the pressure head difference scales quadratically with flow rate (ΔH∝Q2). Across the experimental range of volumetric flow rates from approximately 4.69×10-4 m3/s to 7.80×10-4 m3/s, the required pressure head difference increased non-linearly from around 0.0438 m H2O to 0.1753 m H2O. The dotted trendline effectively captures this quadratic relationship, confirming that higher fluid velocities require significantly larger pressure drops across the restriction to overcome permanent friction losses and fluid contraction at the vena contracta. While the majority of the data points closely adhere to the fitted trendline, minor deviations observed at lower flow rates can be attributed to experimental uncertainties such as initial manometer fluid stabilization, small parallax errors when reading the scale, or minor flow rate fluctuations during volumetric collection.

Figure 4: Plot of Root of Pressure Head Difference vs Volumetric Flowrate

The graph illustrates the relationship between the volumetric flow and the square root of the differential pressure head. In accordance with Bernoulli’s principle and the standard orifice meter discharge equation, the volumetric flow rate is directly proportional to the square root of the differential pressure head, which predicts a linear relationship where the slope is inversely related to the discharge coefficient (Cd). Across the experimental flow range from approximately 4.69×10-4 m3/s to 7.80×10-4 m3/s, the value of ΔH exhibits a strong, positive linear trend, increasing from 0.21m H2O to 0.42m H2O. At higher flow rates, the experimental data points align very closely with the linear trendline, indicating stable, fully turbulent flow conditions where the discharge coefficient remains virtually constant.

Figure 5: Plot of Coefficient of Orifice Meter vs Pressure Head Difference

Figure 5 shows the discharge coefficient, Cd, plotted against the pressure head difference, ΔH×10² (m H₂O). The trendline shows a mild overall decrease in Cd as ΔH increases, but the data are not perfectly linear. The first data point, taken at the lowest ΔH, gives an unusually high Cd of about 0.96, while the second point drops sharply to around 0.73 well below the general trend. Beyond this, the remaining points settle into a much narrower band, ranging from about 0.78 to 0.81, and show only a slight downward drift as ΔH increases further. This behavior suggests that at very low pressure head differences, the orifice meter readings become less reliable, likely because the small ΔH is harder to measure precisely on the manometer, making the resulting Cd values more sensitive to reading error. Once ΔH becomes large enough, Cd stabilizes near 0.79–0.80, indicating that the meter behaves consistently and predictably at higher flow rates.

Figure 6: Plot of Coefficient of Orifice Meter vs Root of Pressure Head Difference

Figure 6 presents the same Cd data, plotted against √ΔH, which is the variable that appears directly in the orifice meter flow-rate equation. The shape of the trend mirrors Figure 5 closely; the lowest √ΔH point again gives an anomalously high Cd near 0.96, the second point dips to about 0.73, and the rest of the data cluster tightly between 0.78 and 0.81 as √ΔH increases. Since plotting against √ΔH does not change the pattern of scatter or the position of the outliers, this confirms that the irregular behavior at low flow is a property of the coefficient itself, arising from measurement uncertainty and possibly non-ideal flow development through the orifice, rather than being an artefact of how the pressure variable was scaled or plotted. The relatively flat, converged region at higher √ΔH reinforces that Cd approaches a stable value once the flow is well established.

Figure 7: Plot of Coefficient of Orifice Meter vs Reynolds Number

Figure 7 plots Cd against Reynolds number, allowing the same trend to be interpreted in terms of flow regime rather than pressure head. The data again show Cd decreasing from a high of about 0.96 at the lowest Reynolds number (around 13,000) down to a fairly constant value of roughly 0.79–0.80 once Re exceeds about 18,000–19,000. This is consistent with known orifice meter behavior. At low Reynolds numbers, the velocity profile and the vena contracta downstream of the orifice have not fully developed, causing Cd to be more variable and flow-dependent, whereas at higher Re the flow becomes fully turbulent and Cd approaches an asymptotic, essentially constant value. The R² value of only 0.1456 shows that a straight-line fit poorly represents the actual relationship the trendline is heavily skewed by the two Reynolds number outliers, while the majority of points at higher Re are tightly clustered and nearly flat.

# 5. Conclusion

This experiment set out to calibrate the orifice meter by empirically determining its coefficient of discharge and characterizing how the pressure head difference responds to changes in volumetric flow rate. Using Bernoulli's equation together with the continuity equation as the theoretical basis, the ideal discharge predicted by these models was compared against the actual flow rates measured across eight separate flow rates.

The results obtained largely supported the underlying fluid mechanics theory. As expected, the pressure head difference rose in a parabolic fashion with increasing flow rate, and this nonlinear relationship became a straight line once the flow rate was plotted against the square root of the pressure head, exactly as predicted by the orifice discharge equation. Since the Reynolds numbers throughout the trials stayed within a fully turbulent range of roughly 13,866.43 to 23,110.72, the flow conditions were suitable for evaluating how the discharge coefficient behaves under high-inertia flow. The computed values of Cd fell between 0.73 and 0.96, with most of the higher-flow readings converging tightly around 0.79 to 0.81.

Ideally, once turbulence is fully established, Cd is expected to hold at a single, constant value regardless of flow rate. In practice, however, the calibration data showed some spread and departure from this flat ideal, most visibly at the lowest flow setting. This scatter can reasonably be attributed to ordinary sources of experimental error, such as unavoidable frictional and eddy losses generated as the fluid passed through the vena contracta.

Even accounting for this scatter, the calibration curves produced still give a usable and representative picture of how this particular orifice meter performs. The average discharge coefficient obtained from the stabilized, high Reynolds number readings offers a dependable basis for estimating unknown flow rates in future work, meaning the experiment met its intended goal of calibrating the meter for practical use.

# References

[1] E. J. Finnemore and J. B. Franzini, Fluid Mechanics with Engineering Applications, 10th ed. Boston: McGraw-Hill, 2001.

[2] Mott, R. L., 2000. Applied Fluid Mechanics, 5th edition; Prentice-Hall Inc., New Jersey.

[3] R. H. Perry and D. W. Green, Perry's Chemical Engineers' Handbook, 9th ed. New York, NY, USA: McGraw-Hill Education, 2018.

[4] The Engineering ToolBox, "Steel Pipes - Schedule 40," 2003. [Online]. Available: https://www.engineeringtoolbox.com/steel-pipes-dimensions-d_43.html. [Accessed: Jul. 23, 2026].

# List of Symbols

| Symbol | Meaning | Unit (SI) |
| --- | --- | --- |
| A | Cross-sectional area of the pipe | m² |
| Ao | Cross-sectional area of the orifice | m² |
| Cd | Coefficient of Discharge | Dimensionless |
| D | Internal diameter of the main pipe | m |
| Do | Orifice diameter | m |
| g | Acceleration due to gravity | m/s² |
| P1, P2 | Static pressure at points 1 and 2 | Pa |
| Q | Experimental volumetric flow rate | m³/s |
| Qth | Theoretical volumetric flow rate | m³/s |
| Rm | Manometer reading difference | m |
| Re | Reynolds Number | Dimensionless |
| t | Time interval | s |
| V | Volume of water | m³ |
| v | Average fluid velocity | m/s |
| v1, v2 | Fluid velocities at points 1 (upstream) and 2 (vena contracta) | m/s |
| z1, z2 | Elevation heads at points 1 and 2 | m |
| ΔH | Pressure head difference | m |
| μw | Dynamic viscosity of water | Pa⋅s |
| ρ | Density | kg/m³ |
| ρw | Density of water | kg/m³ |
| ρCCl4 | Density of Carbon Tetrachloride (CCl4) | kg/m³ |

# Appendices

## Appendix A

### Observed Data

Room Temperature = 29℃

Water Temperature = 29℃

Nominal Pipe Diameter = 2 inch

Schedule Number = 40

Orifice Diameter, Do = 1 inch = 0.0254 m

Diameter of the pipe, D = 2.067 inch = 0.0525 m [4]

Table 2: Observed data table for volume of water, time interval, manometer reading

| No. of Observation | Time Interval, t (s) | Volume of Water (L) | Manometer Reading |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Left (cm) | Right (cm) | Difference, Rm (cm) |
| 1 | 10.66 | 5 | 32.3 | 25.0 | 7.3 |
| 2 | 9.90 |  | 36.4 | 21.5 | 14.9 |
| 3 | 8.29 |  | 37.7 | 20.0 | 17.7 |
| 4 | 7.53 |  | 39.9 | 18.5 | 21.4 |
| 5 | 7.25 |  | 40.6 | 17.4 | 23.2 |
| 6 | 7.00 |  | 41.4 | 16.4 | 25.0 |
| 7 | 6.72 |  | 43.0 | 15.0 | 28.0 |
| 8 | 6.41 |  | 43.6 | 14.4 | 29.2 |

## Appendix B

### Calculated Data

Density of water at 29℃, ρw =995.95 kg/m3 [3]

Viscosity of water at 29℃, μw = 8.1449×10-4 Pa⋅s [3]

Density of CCl4 at 29℃, ρCCl4=1594 kg/m3 [3]

Area of Pipe, A = 2.165×10-3 m2

Table 3: Table for Calculated Data of Volumetric Flow Rate, Velocity and Reynolds Number

| No. of Observation | Time Interval, t (s) | Volume of Water, V×103 (m3) | Volumetric Flow Rate, Q×104 (m3/s) | Velocity, v (m/s) | Reynolds number, Re |
| --- | --- | --- | --- | --- | --- |
| 1 | 10.66 | 5 | 4.69 | 0.217 | 13866.43 |
| 2 | 9.90 |  | 5.05 | 0.233 | 14957.77 |
| 3 | 8.29 |  | 6.03 | 0.279 | 17846.61 |
| 4 | 7.53 |  | 6.64 | 0.307 | 19644.11 |
| 5 | 7.25 |  | 6.90 | 0.319 | 20414.47 |
| 6 | 7.00 |  | 7.14 | 0.330 | 21188.83 |
| 7 | 6.72 |  | 7.44 | 0.344 | 22019.38 |
| 8 | 6.41 |  | 7.80 | 0.36 | 23110.72 |

Table 4: Table for Calculated Data of Pressure Head Difference, Square Root of Pressure Head Difference and Orifice Coefficient

| No. of Observation | Pressure Head Difference, ∆H×102 (m H2O) | Square root of Pressure head difference ∆H (m H2O) | Volumetric Flow Rate Q×104 (m3/s) | Orifice Coefficient, Cd |
| --- | --- | --- | --- | --- |
| 1 | 4.38 | 0.209 | 4.69 | 0.960 |
| 2 | 8.95 | 0.299 | 5.05 | 0.730 |
| 3 | 10.63 | 0.326 | 6.03 | 0.800 |
| 4 | 12.85 | 0.358 | 6.64 | 0.801 |
| 5 | 13.93 | 0.373 | 6.90 | 0.799 |
| 6 | 15.01 | 0.387 | 7.14 | 0.797 |
| 7 | 16.81 | 0.410 | 7.44 | 0.785 |
| 8 | 17.53 | 0.418 | 7.80 | 0.805 |

## Appendix C

### Sample Calculation

For Observation No. 2,

i. Volumetric flow rate

Volume of water, V = 5.0 ×10-3 m3

Time interval, t = 9.90 s

Volumetric flow rate of water, Q = Vt = 5 ×10-3m39.90 s = 5.05 ×10-4  m3/s

ii. Average Velocity

Pipe Diameter D = 2.067inch = 0.0525 m

Area of Pipe, A = π × D24  = 3.1416×(0.0525)2 m24 = 2.165 ×10-3 m2

Average velocity of water, v = QA = 5.05×104 m3/s2.165 ×10-3 m2= 0.233 m/s

iii. Pressure Drop

Manometer reading,

Left reading = 36.4 cm CCl4

Right reading = 21.5 cm CCl4

Manometer reading difference, Rm = (36.4 – 21.5) cm CCl4

= 14.9 cm CCl4 = 14.9×10-2 m CCl4

Pressure Head difference, ∆H = Rm ×ρCCl4ρw -1

= 14.9×10-2 ×1594995.95-1 m H2O

= 8.95×10-2  m H2O

Square Root of Pressure Head Difference, ΔH = 0.299 √m H2O

iv. Coefficient of Discharge

Orifice Diameter, Do = 1 inch = 0.0254 m

Area of Orifice, Ao =  π ×D024  =    3.1416×(0.0254)24   = 5.067×10-4 m2

Coefficient of discharge, Cd  = QA02g∆H1-(DoD)4

= 5.05 ×10-4  5.067×10-4×√2×9.8×0.08951-(12.067)4  = 0.73

v. Reynolds Number,

Re = Dvρw μw = 0.0525× 0.233×995.95 8.1449×10-4 = 14957.77

Marking scheme: Long report on ‘CALIBRATION OF ORIFICE METER’

Name: Shahriar Hossain Shohan

Student number: 2302042

| Assessment Component | Weight (%) | Obtained Marks |
| --- | --- | --- |
| Summary | 10% |  |
| Experimental Set up | 15% |  |
| Result and Discussion and Conclusion | 25% |  |
|  | Subtotal (50%) |  |
| Introduction and Theory | 15% |  |
| Reference and Nomenclature | 05% |  |
| Quality of Tables | 10% |  |
| Quality of Figures | 10% |  |
| Overall Presentation | 10% |  |
| Total | 100% |  |